XiaoMu Robot Programming Edition API Documentation and Programming Guide 1.0

Table of Contents

Chapter 1: Environment Preparation	2

1.1 Overview	2

1.2 Hardware Preparation	2

1.3 Connecting to Wi-Fi	2

1.4 Obtaining the Robot’s IP Address	3

1.5 Remote Access to the Robot	3

1.5.1 Login Credentials	3

1.5.2 Logging in with SSH	3

1.5.3 Transferring Files with SFTP	4

1.6 Programming Environment Description	5

1.6.1 Python Version	5

1.6.2 ROS 2 Environment	5

1.6.3 Workspace Structure	5

1.7 Programming Examples Description	6

1.8 Building and Running	7

1.8.1 Building the Workspace	7

1.8.2 Sourcing the Environment	8

1.8.3 Running Example Programs	8

1.9 Creating Custom Programs	8

1.9.1 Adding a New Python File	9

1.9.2 Adding a New Launch Script	9

1.9.3 Modifying setup.py to Add Entry Point	10

1.9.4 Modifying package.xml to Add Dependencies	10

1.10 Important Notes	11

1.10.1 Program Conflict Handling	11

1.10.2 System Environment Notes	11

1.10.3 System Maintenance Reminders	11

1.11 Troubleshooting	12

1.12 Environment Setup Checklist	12

Chapter 2: ROS 2 Message Interface and API Reference	13

2.1 Overview	13

2.2 Message File Description	13

2.3 Sensor Messages (Subscribe)	14

2.3.1 Temperature & Humidity Sensor (Sensor)	14

2.3.2 IMU Sensor (Imu)	15

2.3.3 Touch Sensor (Touch)	15

2.3.4 Speech Recognition (Listen)	15

2.3.5 RFID Reader (Rifd)	16

2.3.6 Battery Status (BatteryStatus)	16

2.3.7 Vision Recognition Result (Saw)	17

2.4 Control Messages (Publish)	17

2.4.1 Speech Output (Speak)	17

2.4.2 Motion Control (Action)	18

2.4.3 Vision Control (See)	19

2.5 Message Interface Summary Table	19

2.6 API Cheat Sheet	19

Chapter 3: Integrated Application Development	20

3.1 Overview	20

3.2 Development Process	20

3.3 Integrated Application Examples	21

3.3.1 Speech Output (`xiaomu_speak.py`)	21

3.3.2 Touch Interaction (`xiaomu_touch.py`)	21

3.3.3 Sensor Reading (`xiaomu_sense.py`)	21

3.3.4 Motion Control (`xiaomu_move.py`)	22

3.3.5 Vision Recognition (`xiaomu_vision.py`)	22

3.3.6 Intelligent Chat (`xiaomu_chat.py`)	22

3.3.7 Basic Sensor Reading (`simple.py`)	23

3.4 Example Difficulty Gradient Description	23

3.5 Running Multiple Nodes Simultaneously	23

3.6 Adding Custom Programs	24

3.6.1 Adding a New Python File	24

3.6.2 Modifying setup.py to Add Entry Point	24

3.6.3 Creating a Launch File	24

3.6.4 Building and Running	24

3.7 Development Best Practices	24

3.8 Common Issues	25

3.9 Chapter Summary	25

Appendix A: Common Commands Quick Reference	25

Appendix B: Directory Structure	26

Appendix C: Copyright and Confidentiality Statement	27

C.1 Copyright Notice	27

C.2 Confidentiality Statement	27

C.3 Consequences of Violation	27

Chapter 1: Environment Preparation

1.1 Overview

XiaoMu Robot Programming Edition is equipped with the Linux operating system. Users can access the robot remotely via the network and conduct secondary development using Python 3.9 and the ROS 2 framework（IRON）. This chapter guides you through the preparation of the programming environment.

1.2 Hardware Preparation

Before you start programming, ensure the following devices are ready:

- XiaoMu Robot (powered on)

- Router (supports Wi-Fi)

- A computer connected to the network (Windows / macOS / Linux)

---

1.3 Connecting to Wi-Fi

XiaoMu Robot needs to connect to the local area network via Wi-Fi so that your computer can access it remotely.

Steps:

1. Start XiaoMu Robot by following the steps in the XiaoMu user manual.

2. Enter XiaoMu’s network configuration mode (refer to the product manual for specific operations).

3. Select the Wi-Fi network you want to connect to and enter the password.

4. Wait for the robot to confirm a successful connection.

💡 **Tip**: When the connection is successful, the robot will announce a voice prompt like “Network connected”.

1.4 Obtaining the Robot’s IP Address

The IP address of XiaoMu Robot can be viewed through the router management interface.

Steps:

1. Enter the router’s management address in your computer browser (usually `192.168.1.1` or `192.168.0.1`).

2. Log in with the router administrator username and password.

3. Find the menu such as “Connected Devices”, “DHCP Client List” or “Device Management”.

4. Look for a device named `linaro-alip` or similar in the list.

5. Record the IP address of this device (e.g. `192.168.1.101`).

⚠️ **Note**: The router management interface varies by brand and model. Please refer to the router manual for details.

---

1.5 Remote Access to the Robot

After obtaining the IP address, you can use SSH and SFTP tools to access the robot remotely.

1.5.1 Login Credentials

| Item | Information |

|------|-------------|

| **Username** | `linaro` |

| **Password** | `linaro` |

| **Port** | `22` (default) |

1.5.2 Logging in with SSH

**For Windows users:**

Recommended tools:

- **PuTTY** (lightweight SSH client)

- **MobaXterm** (more comprehensive functions)

- **PowerShell** (built-in for Windows 10/11)

Example with PowerShell:

```powershell

ssh linaro@<ROBOT_IP_ADDRESS>

```

**For macOS / Linux users:**

Open Terminal and run the following command:

```bash

ssh linaro@<ROBOT_IP_ADDRESS>

```

Example:

```bash

ssh linaro@192.168.1.100

```

When connecting for the first time, you will be prompted to confirm the fingerprint. Type `yes` to continue.

1.5.3 Transferring Files with SFTP

SFTP (SSH File Transfer Protocol) is used to transfer files between your computer and the robot.

**Recommended SFTP tools:**

| OS | Recommended Tools |

|------|-------------|

| Windows | WinSCP, MobaXterm, FileZilla |

| macOS | Cyberduck, FileZilla, Terminal |

| Linux | Remmina, FileZilla, Terminal commands |

**SFTP via Terminal:**

```bash

sftp linaro@<ROBOT_IP_ADDRESS>

```

**Using GUI tools (WinSCP as example):**

1. Open WinSCP

2. Select `SFTP` as the file protocol

3. Enter the robot’s IP address as the host name

4. Username `linaro`, password `linaro`

5. Click Login

---

1.6 Programming Environment Description

1.6.1 Python Version

XiaoMu Programming Edition supports **Python 3.9**:

```bash

# Verify Python version after SSH login

python3 --version

# Example output: Python 3.9.2

```

1.6.2 ROS 2 Environment

XiaoMu Robot runs based on the ROS 2 framework. Users need to understand basic ROS 2 concepts (nodes, topics, messages, etc.).

**Common ROS 2 commands:**

```bash

# List ROS 2 nodes

ros2 node list

# List topics

ros2 topic list

# Show message definition

ros2 interface show robot_interfaces/msg/Speak

```

1.6.3 Workspace Structure

The user workspace is located at `/home/linaro/tm_study/`, with the following structure:

```bash

/home/linaro/tm_study/

├── src/                           # Source code directory

│   └── example/                   # Example package

│       ├── example/               # Python module directory

│       │   ├── __init__.py

│       │   ├── simple.py          # Example: Basic sensor reading

│       │   ├── xiaomu_speak.py    # Example: Audio output

│       │   ├── xiaomu_touch.py    # Example: Touch interaction

│       │   ├── xiaomu_sense.py    # Example: Sensor reading

│       │   ├── xiaomu_move.py     # Example: Motion control

│       │   ├── xiaomu_chat.py     # Example: Intelligent chat

│       │   └── xiaomu_vision.py   # Example: Vision recognition

│       ├── setup.py               # Package installation config

│       └── package.xml            # Package dependency declaration

├── launch/                        # Launch scripts directory

│   ├── simple_launch.py

│   ├── speak_launch.py

│   ├── touch_launch.py

│   ├── sense_launch.py

│   ├── move_launch.py

│   ├── chat_launch.py

│   └── vision_launch.py

└── install/                       # Compiled installation directory

```

**Directory / File Description:**

| Directory / File | Purpose |

|-------------|-------------|

| `src/example/example/` | Python program files (*.py) |

| `launch/` | ROS 2 launch scripts (*.launch.py) |

| `src/example/setup.py` | Package installation configuration, defines entry points |

| `src/example/package.xml` | Package dependency declaration file |

| `install/` | Compiled output directory |

---

1.7 Programming Examples Description

Complete programming examples are provided under the `tm_study` directory to help you get started quickly:

| Example File | Description |

|-------------|-------------|

| `simple.py` | Basic example: Subscribe to all topics including sensors, touch, IMU, vision, speech recognition, etc. |

| `xiaomu_speak.py` | Speech output example: Make XiaoMu speak |

| `xiaomu_touch.py` | Touch interaction example: Respond when head or belly is touched |

| `xiaomu_sense.py` | Sensor example: Read temperature, humidity, IMU data and announce |

| `xiaomu_move.py` | Motion control example: Nod, shake head, wave, etc. |

| `xiaomu_chat.py` | Intelligent chat example: Complete interaction integrating speech recognition, motion, touch |

| `xiaomu_vision.py` | Vision recognition example: Enable vision, recognize face emotions and objects and respond |

**Supporting Launch Scripts (launch directory):**

- `simple_launch.py`

- `speak_launch.py`

- `touch_launch.py`

- `sense_launch.py`

- `move_launch.py`

- `chat_launch.py`

- `vision_launch.py`

---

1.8 Building and Running

1.8.1 Building the Workspace

```bash

cd /home/linaro/tm_study

colcon build --packages-select example

```

Or build all modules directly:

```bash

cd /home/linaro/tm_study

colcon build

```

1.8.2 Sourcing the Environment

```bash

source /home/linaro/tm_study/install/setup.bash

```

1.8.3 Running Example Programs

**Unified launch method using ros2 launch:**

```bash

cd /home/linaro/tm_study

# Basic example

ros2 launch launch/simple_launch.py

# Speech output example

ros2 launch launch/speak_launch.py

# Touch interaction example

ros2 launch launch/touch_launch.py

# Sensor example

ros2 launch launch/sense_launch.py

# Motion control example

ros2 launch launch/move_launch.py

# Intelligent chat example

ros2 launch launch/chat_launch.py

# Vision recognition example

ros2 launch launch/vision_launch.py

```

---

1.9 Creating Custom Programs

1.9.1 Adding a New Python File

User-created custom Python files should be placed in the following directory:

```bash

/home/linaro/tm_study/src/example/example/

```

**Example: Create a custom program**

```bash

# Execute after SSH login

cd /home/linaro/tm_study/src/example/example/

touch my_custom_node.py

```

You can also develop locally on your computer and transfer related files to the corresponding directory via SFTP.

1.9.2 Adding a New Launch Script

User-created launch scripts should be placed in the following directory:

```bash

/home/linaro/tm_study/launch/

```

**Launch Script Template:**

```python

from launch import LaunchDescription

from launch_ros.actions import Node

def generate_launch_description():

return LaunchDescription([

Node(

package='example',

namespace='tmmini',

executable='my_custom_node',

name='my_custom_node'

),

])

```

1.9.3 Modifying setup.py to Add Entry Point

In `/home/linaro/tm_study/src/example/setup.py`, find the `entry_points` section and add your program entry:

```python

entry_points={

'console_scripts': [

'simple = example.simple:main',

'speak = example.xiaomu_speak:main',

'touch = example.xiaomu_touch:main',

'sense = example.xiaomu_sense:main',

'move = example.xiaomu_move:main',

'chat = example.xiaomu_chat:main',

'vision = example.xiaomu_vision:main',

'my_custom_node = example.my_custom_node:main',  # Add your program

],

},

```

**Format Description for entry_points:**

| Part | Description |

|------|-------------|

| `executable_name` | Name used in the `executable` field of the launch file |

| `module_path` | Python module path relative to `/home/linaro/tm_study/src/example/` |

| `function_name` | The `main` function name in the Python program |

1.9.4 Modifying package.xml to Add Dependencies

In `/home/linaro/tm_study/src/example/package.xml`, add third-party Python dependencies:

```xml

<!-- Add Python dependencies -->

<exec_depend>python3-requests</exec_depend>

<exec_depend>python3-numpy</exec_depend>

<exec_depend>python3-opencv</exec_depend>

```

**Common Dependencies:**

| Dependency | Purpose |

|------|-------------|

| `python3-numpy` | Numerical calculation |

| `python3-opencv` | Image processing |

| `python3-requests` | HTTP requests |

💡 **Tip**: After adding dependencies, you need to rebuild the workspace for changes to take effect.

---

1.10 Important Notes

1.10.1 Program Conflict Handling

When users write their own programs, resource conflicts may occur with the robot’s main control program (e.g., occupying the same sensors or actuators), causing abnormal operation of the main control.

**Solutions:**

1. **Stop the custom program**: Press `Ctrl + C` in the terminal to terminate the running user program.

2. **Restart the robot main control**: Reboot the robot to restore normal operation.

💡 **Recommendation**: During development and debugging, avoid occupying key resources required by the main control in your own programs.

1.10.2 System Environment Notes

If you are not familiar with the Linux system, please follow these rules:

- ❌ **Do not modify system parameters** at will

- ❌ **Do not modify system environment variables**

- ❌ **Do not delete or modify key system directories**

- ✅ **Only operate in your own working directory** (`/home/linaro/tm_study/`)

- ✅ **Only use SSH/SFTP for development and file transfer**

1.10.3 System Maintenance Reminders

- **Do not attempt to attack the robot system**: Including but not limited to cracking system permissions, maliciously modifying system files, etc.

- **Do not delete key system files**.

- **Maintain a good development environment together**. Thank you for your cooperation and understanding.

1.11 Troubleshooting

| Issue | Possible Cause | Solution |

|------|-------------|-------------|

| SSH connection failed | Wrong IP or network unreachable | Reconfirm IP via router; check if robot and computer are on the same network |

| Incorrect password | Caps lock or typo | Confirm password is `linaro` (lowercase); check keyboard |

| SFTP transfer failed | Insufficient disk space | Run `df -h` to check space; clean unnecessary files |

| Program runtime error | Missing Python dependencies | Add dependencies in `package.xml` and rebuild |

| Build failed | Wrong `setup.py` configuration | Check if `entry_points` format is correct |

| Launch file not found | Launch file misplaced or unconfigured | Confirm launch file is in `/home/linaro/tm_study/launch/` |

| Conflict with main control | Resource occupation conflict | Stop custom program or reboot robot |

| Cannot find robot IP | Router does not show device | Confirm robot is connected to Wi-Fi; refresh router page |

---

1.12 Environment Setup Checklist

After completing this chapter, confirm the following items:

- [ ] Robot powered on and connected to Wi-Fi

- [ ] Obtained robot IP via router management page

- [ ] Successfully logged into robot via SSH (username/password: linaro/linaro)

- [ ] Able to transfer files to robot via SFTP

- [ ] Confirmed Python 3.9 environment is normal

- [ ] Understood basic ROS 2 concepts

- [ ] Understood workspace structure (`/home/linaro/tm_study/`)

- [ ] Understood Python file location (`src/example/example/`)

- [ ] Understood Launch file location (`/home/linaro/tm_study/launch/`)

- [ ] Understood how to add custom program entry (`setup.py`)

- [ ] Understood how to add third-party dependencies (`package.xml`)

- [ ] Understood how to build and run programs

- [ ] Familiar with provided programming examples

---

Chapter 2: ROS 2 Message Interface and API Reference

2.1 Overview

All functional modules of XiaoMu Robot communicate through ROS 2 message interfaces. Users can obtain sensor data, control robot motion, output speech, etc., by subscribing and publishing specific topics.

This chapter introduces the message interfaces provided by XiaoMu Robot. **For detailed definitions of messages (including field types, constant values, etc.), please refer to the results queried by the `ros2 interface show` command.**

> 💡 **How to query message definitions**:

> ```bash

> # List all available messages

> ros2 interface list | grep robot_interfaces

>

> # Show specific message definition

> ros2 interface show robot_interfaces/msg/Speak

> ros2 interface show robot_interfaces/msg/Touch

> ros2 interface show robot_interfaces/msg/Action

> ```

---

2.2 Message File Description

XiaoMu Robot uses the custom message package `robot_interfaces`, and you need to import the corresponding message types in Python scripts:

```python

from robot_interfaces.msg import Sensor, Imu, Touch, Speak, Listen, Action, Servo, See, Saw, Emotion, Led, Rifd, BatteryStatus

```

**Message Package Dependency**: Add the following in `package.xml`:

```xml

<depend>robot_interfaces</depend>

```

---

2.3 Sensor Messages (Subscribe)

The following messages are used to obtain sensor data by subscribing to corresponding topics.

| Module | Topic Name | Message Type | Description |

|------|-------------|-------------|-------------|

| Temperature & Humidity | `/sensor` | `Sensor` | Get temperature and humidity data |

| IMU | `/imu` | `Imu` | Get attitude and motion data |

| Touch | `/touch` | `Touch` | Get touch status |

| Speech Recognition | `/listen` | `Listen` | Get speech recognition text |

| RFID | `/rifd` | `Rifd` | Get RFID card UID |

| Battery | `/battery` | `BatteryStatus` | Get battery status |

| Vision Result | `/saw` | `Saw` | Get vision recognition results |

2.3.1 Temperature & Humidity Sensor (Sensor)

**Topic**: `/sensor`

**Usage Example**:

```python

def sensor_callback(self, msg):

temp = msg.temperature

humidity = msg.humidity

self.get_logger().info(f'Temperature: {temp:.1f}°C, Humidity: {humidity:.1f}%')

if temp > 35:

self._speak("It's so hot, please cool me down")

```

---

2.3.2 IMU Sensor (Imu)

**Topic**: `/imu`

**Usage Example**:

```python

import math

def imu_callback(self, msg):

ax, ay, az = msg.accel_x, msg.accel_y, msg.accel_z

pitch = math.atan2(ax, az) * 180 / math.pi

self.get_logger().info(f'Pitch angle: {pitch:.1f}°')

```

---

2.3.3 Touch Sensor (Touch)

**Topic**: `/touch`

**Description**: A value of 0 for `head_touch_state` and `belly_touch_state` means no touch, 1 means touched.

**Usage Example**:

```python

def touch_callback(self, msg):

if msg.head_touch_state == 1:

self._speak("Hehe, patting my head feels so good")

self._nod()

if msg.belly_touch_state == 1:

self._speak("Hee hee, that tickles")

self._shake_head()

```

---

2.3.4 Speech Recognition (Listen)

**Topic**: `/listen`

**Usage Example**:

```python

def listen_callback(self, msg):

text = msg.data.lower()

if 'hello' in text:

self._speak("Hello! Nice to see you")

self._wave()

elif 'goodbye' in text:

self._speak("Goodbye, see you next time")

self._wave()

```

---

2.3.5 Battery Status (BatteryStatus)

**Topic**: `/battery`

**Usage Example**:

```python

def battery_callback(self, msg):

if msg.status == 1:

self.get_logger().info('Charging')

elif msg.status == 2:

self.get_logger().info('Battery full')

else:

self.get_logger().info(f'Current battery: {msg.percentage}%')

```

---

2.3.6 Vision Recognition Result (Saw)

**Topic**: `/saw`

**Description**: The message contains two lists, `persons` and `things`, storing detected people and objects respectively.

**Usage Example**:

```python

def saw_callback(self, saw_data):

for person in saw_data.persons:

if person.emotion == 1:

self._wave()

self._speak("You look happy!")

for thing in saw_data.things:

if thing.type == "cat":

self._speak("What a cute cat")

```

---

2.4 Control Messages (Publish)

The following messages are used to control the robot by publishing to corresponding topics.

| Module | Topic Name | Message Type | Description |

|------|-------------|-------------|-------------|

| Speech Output | `/speak` | `Speak` | Broadcast text |

| Motion Control | `/action` | `Action` | Control servos and LEDs |

| Vision Control | `/eye` | `See` | Start/Stop vision recognition |

2.4.1 Speech Output (Speak)

**Topic**: `/speak`

**Usage Example**:

```python

def _speak(self, text):

msg = Speak()

msg.text = text

self.speak_pub.publish(msg)

```

Note: The output text must match the robot’s current system language, otherwise abnormal broadcast will occur.

---

2.4.2 Motion Control (Action)

**Topic**: `/action`

**Description**:

- `Action` message includes `time` (execution time), `type` (action type), `servos` (servo list), `leds` (LED list)

- For action type definitions, check constants via `ros2 interface show`

**Usage Example**:

```python

def _wave(self):

action = Action()

action.time = 0.5

action.type = 0

servo = Servo()

servo.index = 4

servo.angle = 45

action.servos.append(servo)

self.action_pub.publish(action)

```

```python

def reset(self, duration=1.0):

action = Action()

action.time = float(duration)

action.type = 2  # Reset type

self.action_pub.publish(action)

```

---

2.4.3 Vision Control (See)

**Topic**: `/eye`

**Usage Example**:

```python

def start_vision(self):

see = See()

see.command = 1

self.eye_pub.publish(see)

self._speak("Vision recognition enabled")

```

---

2.5 Message Interface Summary Table

| Module | Topic Name | Message Type | Direction | Description |

|------|-------------|-------------|-------------|-------------|

| Temperature & Humidity | `/sensor` | `Sensor` | Subscribe | Get temperature and humidity data |

| IMU | `/imu` | `Imu` | Subscribe | Get attitude and motion data |

| Touch | `/touch` | `Touch` | Subscribe | Get touch status (0/1) |

| Speech Recognition | `/listen` | `Listen` | Subscribe | Get recognition text |

| Battery | `/battery` | `BatteryStatus` | Subscribe | Get battery status |

| Vision Result | `/saw` | `Saw` | Subscribe | Get vision recognition results |

| Speech Output | `/speak` | `Speak` | Publish | Broadcast text |

| Motion Control | `/action` | `Action` | Publish | Control servos and LEDs |

| Vision Control | `/eye` | `See` | Publish | Start/Stop vision |

---

2.6 API Cheat Sheet

| Function | Operation | Interface |

|------|-------------|-------------|

| Make XiaoMu speak | Publish | `Speak` |

| Control servos | Publish | `Action` + `Servo` |

| Reset servos | Publish | `Action` (Reset type) |

| Start vision | Publish | `See` (start command) |

| Read temperature & humidity | Subscribe | `Sensor` |

| Read attitude | Subscribe | `Imu` |

| Read touch | Subscribe | `Touch` |

| Read speech | Subscribe | `Listen` |

| Get vision results | Subscribe | `Saw` |

> 💡 **Important Note**: Field types, constant values, servo indexes, etc., in messages are subject to the actual output of the `ros2 interface show` command.

---

Chapter 3: Integrated Application Development

3.1 Overview

After mastering the message interfaces in Chapter 2, this chapter guides you to comprehensively use each module to develop robot applications with practical functions. By combining speech, motion, sensors, vision and other capabilities, you can create rich interactive experiences.

---

3.2 Development Process

Developing a complete robot application usually follows these steps:

1. **Define functional requirements**: Clarify the scenarios and behaviors the robot needs to respond to

2. **Design interaction logic**: Plan trigger conditions, response actions, state transitions

3. **Write node code**: Implement subscription and publishing logic

4. **Add launch scripts**: Create launch files for easy startup

5. **Build and test**: Compile workspace and run verification

---

3.3 Integrated Application Examples

The following examples are based on existing files in the `tm_study/src/example/example/` directory, arranged from simple to complex. You can study and refer according to your needs.

3.3.1 Speech Output (`xiaomu_speak.py`)

**Function**: Basic example of making XiaoMu speak, demonstrating the most basic speech output capability.

**Interfaces involved**: `Speak` (speech output)

**Launch Command**:

```bash

ros2 launch launch/speak_launch.py

```

---

3.3.2 Touch Interaction (`xiaomu_touch.py`)

**Function**: XiaoMu responds with speech and motion when head or belly is touched.

**Interfaces involved**: `Touch` (touch detection), `Speak` (speech output), `Action` (motion control)

**Launch Command**:

```bash

ros2 launch launch/touch_launch.py

```

---

3.3.3 Sensor Reading (`xiaomu_sense.py`)

**Function**: Read temperature, humidity and IMU sensor data and announce.

**Interfaces involved**: `Sensor` (temperature & humidity), `Imu` (attitude sensor), `Speak` (speech output)

**Launch Command**:

```bash

ros2 launch launch/sense_launch.py

```

---

3.3.4 Motion Control (`xiaomu_move.py`)

**Function**: Control XiaoMu to perform various actions, including nod, shake head, wave, reset, etc.

**Interfaces involved**: `Action` (motion control), `Speak` (speech feedback)

**Launch Command**:

```bash

ros2 launch launch/move_launch.py

```

---

3.3.5 Vision Recognition (`xiaomu_vision.py`)

**Function**: Enable vision recognition, recognize face emotions and objects and respond.

**Interfaces involved**: `See` (vision control), `Saw` (vision result), `Speak` (speech output), `Action` (motion control)

**Launch Command**:

```bash

ros2 launch launch/vision_launch.py

```

---

3.3.6 Intelligent Chat (`xiaomu_chat.py`)

**Function**: Complete interactive example integrating speech recognition, motion control, touch interaction.

**Interfaces involved**: `Listen` (speech recognition), `Touch` (touch detection), `Speak` (speech output), `Action` (motion control)

**Launch Command**:

```bash

ros2 launch launch/chat_launch.py

```

---

3.3.7 Basic Sensor Reading (`simple.py`)

**Function**: Subscribe to all sensor topics, demonstrate data reading methods, suitable as a reference template for developing custom nodes.

**Interfaces involved**: `Sensor`, `Imu`, `Touch`, `Listen`, `Saw`, `Rifd`, `BatteryStatus`

**Launch Command**:

```bash

ros2 launch launch/simple_launch.py

```

---

3.4 Example Difficulty Gradient Description

| Difficulty | Example | Core Learning Objective |

|------|-------------|-------------|

| ★☆☆ | `xiaomu_speak.py` | Master message publishing (Speak) |

| ★☆☆ | `xiaomu_touch.py` | Master message subscription (Touch) + simple response |

| ★★☆ | `xiaomu_sense.py` | Master sensor data processing |

| ★★☆ | `xiaomu_move.py` | Master motion control (Action + Servo) |

| ★★★ | `xiaomu_vision.py` | Master vision module control and result processing |

| ★★★ | `xiaomu_chat.py` | Comprehensive use of multiple modules for complete interaction |

| ★★★ | `simple.py` | Understand all message interfaces, use as development template |

---

3.5 Running Multiple Nodes Simultaneously

You can launch multiple nodes at the same time using one launch file, referring to the format of existing files in the `launch` directory.

---

3.6 Adding Custom Programs

3.6.1 Adding a New Python File

Create a new `.py` file in the `/home/linaro/tm_study/src/example/example/` directory, referring to the structure of existing examples.

3.6.2 Modifying setup.py to Add Entry Point

Add the entry of the new program in `entry_points` of `setup.py`:

```python

'my_node = example.my_node:main',

```

3.6.3 Creating a Launch File

Create the corresponding `_launch.py` file in the `/home/linaro/tm_study/launch/` directory.

3.6.4 Building and Running

```bash

cd /home/linaro/tm_study

colcon build --packages-select example

source install/setup.bash

ros2 launch launch/my_node_launch.py

```

---

3.7 Development Best Practices

| Practice | Description |

|------|-------------|

| Single Responsibility | Each node is responsible for only one clear functional module |

| Loose Coupling | Nodes communicate via topics, avoid direct dependencies |

| Parameterization | Use `declare_parameter` and `get_parameter` to make nodes configurable |

| Log Level | Use `debug/info/warn/error` levels for logging |

| Exception Handling | Catch exceptions to avoid node crash |

---

3.8 Common Issues

| Issue | Solution |

|------|-------------|

| No response after node startup | Run `source install/setup.bash` to load environment |

| No topic data received | Check if `namespace` in launch file is set to `tmmini` |

| Message not found during build | Confirm `<depend>robot_interfaces</depend>` exists in `package.xml` |

| Multiple node conflicts | Stop other nodes or use different namespaces |

---

3.9 Chapter Summary

The `tm_study` workspace provides programming examples from simple to complex, covering speech, motion, sensors, touch, vision and other modules. It is recommended to learn in the following path:

1. Start with `xiaomu_speak.py` to understand the basic publish-subscribe model

2. Learn touch, sensor, motion control examples in sequence

3. Try comprehensive examples such as vision recognition and intelligent chat

4. Refer to `simple.py` to understand all interfaces and start developing your own nodes

The launch scripts corresponding to each example are located in the `/home/linaro/tm_study/launch/` directory, one-to-one with the example files.

---

Appendix A: Common Commands Quick Reference

```bash

# ROS 2 environment setup (executed by default when SSH login)

source /opt/ros2/iron/setup.bash

# Project environment setup

source /home/linaro/tm_study/install/setup.bash

# Build

cd /home/linaro/tm_study && colcon build --packages-select example

# List topics

ros2 topic list

# Show message definition

ros2 interface show robot_interfaces/msg/MESSAGE_NAME

# Launch example

ros2 launch launch/EXAMPLE_NAME_launch.py

```

---

Appendix B: Directory Structure

```bash

/home/linaro/tm_study/

├── src/example/example/      # Python program directory

│   ├── simple.py

│   ├── xiaomu_speak.py

│   ├── xiaomu_touch.py

│   ├── xiaomu_sense.py

│   ├── xiaomu_move.py

│   ├── xiaomu_chat.py

│   └── xiaomu_vision.py

├── launch/                   # Launch script directory

│   ├── simple_launch.py

│   ├── speak_launch.py

│   ├── touch_launch.py

│   ├── sense_launch.py

│   ├── move_launch.py

│   ├── chat_launch.py

│   └── vision_launch.py

└── install/                  # Compiled output directory

```

Appendix C: Copyright and Confidentiality Statement

C.1 Copyright Notice

This document and all its contents (including but not limited to text, images, code examples, interface definitions, etc.) are the copyright property of Timu Technology.

No individual or organization may, without the prior written permission of Timu Technology:

- Copy, reproduce, or distribute all or part of this document

- Publish the contents of this document on the internet, forums, social media, or any other public channels

- Use this document for commercial purposes

C.2 Confidentiality Statement

This document contains technical information, communication interface definitions, and development methods related to Timu Technology products, which constitute confidential information of Timu Technology.

As a user of the XiaoMu Robot Programming Edition, you agree to:

- Do not distribute: Do not provide this document or any technical information contained herein to any third party in any form

- Do not disclose: Do not publish the contents of this document or any technical implementation based on it on the internet, GitHub, technical forums, or other public channels

- Internal use only: This document is intended solely for your own or your team's internal use for secondary development of the XiaoMu robot

C.3 Consequences of Violation

Unauthorized dissemination or public disclosure of the contents of this document may constitute infringement. Timu Technology reserves the right to pursue legal remedies.

Thank you for your understanding and cooperation. For any questions, please contact Timu Technology official support.